KNX Association

Serial Data Transmission and KNX Protocol

Table of Contents

1	Fundamentals of Serial Data Transmission	4

1.1	Introduction and terminology	4

1.1.1	What is serial data transmission?	4

1.1.2	Difference Operation Level / Process Level / Field Level	4

1.1.3	Classification of serial buses	5

1.1.3.1	Frequency Division Multiplexing (FDM)	5

1.1.3.2	Time Division Multiplexing (TDM)	6

1.1.3.3	Types of modulation	6

1.1.3.4	Baseband transmission:	7

2	The OSI Reference Model (ISO/IEC 7498)	8

2.1	Introduction to the OSI reference model	8

2.1.1	Why are Standards necessary?	8

2.1.2	Objective of the OSI reference model	8

2.2	The principle of the OSI reference model	8

2.2.1	The 7 Layers of the OSI Reference Model	9

2.2.2	The 2 types of communication	10

2.2.2.1	Connection-oriented communication:	10

2.2.2.2	Connectionless communication	10

2.2.3	Protocol Data Units (PDUs)	11

3	Principle functions of the Layers – Implementation in KNX	12

3.1	Application Layer	12

3.2	Presentation Layer (Not used in KNX)	13

3.3	Session Layer (Not used in KNX)	13

3.4	Transport Layer	14

3.4.1	Role 1: Actual transport of the data	14

3.4.2	Role 2: Serving the Application Layer	15

3.5	Network Layer	16

3.5.1	Device behavior	16

3.6	Link Layer	17

3.6.1	Some examples	18

3.6.2	Telegram structure according to cEMI	20

3.6.2.1	1 - Message Code	21

3.6.2.2	2 - Additional Info Length + 3 - Additional Information	21

3.6.2.3	4 – Control field 1	22

3.6.2.4	5 – Control field 2	22

3.6.2.5	6-7 – Source Address High + Source Address Low (SAH + SAL)	23

3.6.2.6	8-9 – Destination Address High + Destination Address Low (DAH + DAL)	23

3.6.2.7	10 – NPDU	23

3.6.2.7.1	L = 0: control data	24

3.6.2.7.2	L > 0: data package	26

3.6.2.7.3	The standard  APCI Codes  and their meaning	29

3.6.2.7.4	Detailed description of some APCIs	30

3.6.2.8	Examples	35

3.6.3	TL Session example: Downloading a KNX Virtual application program by means of ETS	45

3.6.4	Acknowledgment of telegrams by Link Layer	57

3.6.4.1	Positive Acknowledgement (LL_ACK)	58

3.6.4.2	Negative Acknowledgement (LL_NAK)	58

3.6.4.3	Busy Acknowledgement (LL_BUSY)	58

3.6.4.4	Multiple LL Acknowledgements	58

3.6.4.5	Priority LL Acknowledgements	59

3.6.4.6	LL Acknowledgements in case there are couplers	60

3.6.4.6.1	Sender and receiver are in the same line	60

3.6.4.6.2	Sender and receiver are on opposite side of a coupler	60

3.6.4.6.3	Sender and receiver are on opposite side of two couplers	60

3.7	Physical Layer	61

3.7.1	Serial characters	61

3.7.2	Transmission time on TP1	62

3.7.3	Collision Avoidance: Bus access control on TP1	63

3.7.3.1	Example	64

3.7.4	Signal shape on TP1	65

3.7.4.1	Active vs. passive signals	65

4	Appendix: Further examples of cEMI telegrams	69

# Fundamentals of Serial Data Transmission

## Introduction and terminology

### What is serial data transmission?

Serial data transmission is the process of sending data one bit at a time, sequentially, over the same communication channel. This is in contrast to parallel communication, where multiple bits (usually 8 bits) are sent simultaneously on different channels (wires, frequency channels) within the same cable, radio path, and synchronized to a clock.

### Difference Operation Level / Process Level / Field Level

The operation level handles large data volumes but on the other hand requires no time-critical data transmissions.

Relatively small amounts of data are processed on the field level but uncontrollable delays in data transmission cannot be tolerated at this level of the automated installation (= time-critical).

The networks used in automated building installations are typical field bus systems (sensor-actuator-bus), incl. KNX.

Figure 1: KNX is a typical field bus system

### Classification of serial buses

Serial busses are classified according to their methods of data transmission and bus access:

Figure 2: Classification of Serial buses

#### Frequency Division Multiplexing (FDM)

Frequency Division Multiplexing is a technique by which the total bandwidth available in a communication medium is divided into a series of non-overlapping frequency sub-bands, each of which is used to carry out a separate signal.

The messages to be transmitted modulate the carrier frequencies, either by amplitude or by frequency modulation.

It is because of this modulation that several channels (different frequencies), each of which allows data traffic independently of the others, can be transmitted on one bus cable. This means that more than one device can access the bus at one given instance.

An example of Frequency Division Multiplexing is cable television, in which many television channels are carried simultaneously on a single cable.

#### Time Division Multiplexing (TDM)

In TDM, certain time slices are allocated to each message or channel, i.e., the bus devices may only use the bus one after the other. It is therefore that access to the bus must be controlled in order to prevent simultaneous bus access (= collision) of several bus devices. Examples of Time Division Multiplexing are:

centralized bus allocation (Master/Slave approach)

decentralized bus allocation (e.g., Token Passing, Flying Master)

combination of both access control methods (e.g., PROFIBUS)

CSMA/CD	(Carrier Sense Multiple Access with Collision Detection e.g., Ethernet)

CSMA/CA	(Carrier Sense Multiple Access with Collision Avoidance e.g., KNX)

#### Types of modulation

The user signal (= no modulation imposed on yet) will change with the type of modulation imposed on it. The shape of the signal will in other words change with the type of modulation used. The figure below represents the change of the shape of the signal with different modulation types.

Figure 3: Change of shape of signal with modulation (Bit representation)

Modulation is used both for Frequency Division and for Time Division Multiplexing.

#### Baseband transmission:

In baseband transmission, the data bits are directly converted into signals. The shape of the signal will change with baseband transmission. In KNX TP, the binary information is transmitted in the form of rectangular pulses where a pulse represents a “0” and where no pulse represents a “1” on the bus.

In the figure below, you can find the typical encoding schemes of baseband transmission. You can also see here that 1), 2) and 4) only work with an additionally transmitted clock signal, while 3) and 5) have the clock integrated in the signal.

Example: Consider the bit sequence 00101100

Figure 4: Change of shape of signal with Baseband transmission (Bit representation)

# The OSI Reference Model (ISO/IEC 7498)

## Introduction to the OSI reference model

### Why are Standards necessary?

When data processing systems and communication networks of different manufacturers were first introduced, there was no compatibility between them. The interconnection of networks was therefore difficult and only possible with great efforts. The different networks on the market were designed for very different purposes in the industry and business sectors. Each network was a self-contained system closed to communication with another.

### Objective of the OSI reference model

The International Standardization Organization (ISO) in the late 1970s therefore decided to formulate a standard architecture for computer networks and information processing. The aim was to:

develop a standardized model in data communications on the basis of which distributed networks and systems could talk to each other;

achieve compatibility among networks without insisting on implementation of the complete model for each network;

achieve open communication (Open Systems Interconnection OSI) among all users of a heterogeneous computer network.

## The principle of the OSI reference model

OSI generally specifies the functions required for data communication and the interaction between these functions. The OSI model stipulates that higher-level functions build on lower-level functions according to well-defined rules. The result is therefore a hierarchical concept consisting of layers.

### The 7 Layers of the OSI Reference Model

| - Layer 7 - Application Layer | - Can be freely defined by the application. |
| --- | --- |
| - Layer 6 - Presentation Layer | - Data format conversions, data compression, message coding (not used in KNX). |
| - Layer 5 - Session Layer | - Control of logical connections, link to the application process, buffer management (not used in KNX) |
| - Layer 4 - Transport Layer | - End-to-end control, transport control. Layer 4 is responsible for the transport of the entire data volume (i.e., of all the blocks or packets); packet organization, multiplexing. |
| - Layer 3 - Network Layer | - Determination of suitable paths for data transmission and switching the links involved (routing), packet control, transport protocol through the entire network. |
| - Layer 2 - (Data) Link layer | - Layer for connections and procedures; line protocol, synchronization, data consistency, error handling. |
| - Layer 1 - Physical Layer | - Generation of electrical signals, depending on the transmission medium used; connector pin assignment, control signals, bit coding. - Conversion of digital signals into analog and vice versa. |

In KNX, each layer will contribute to a part of the telegram when a device generates a telegram and will decode part of the telegram when receiving. In other words:

When creating a telegram: each layer will insert its relevant information before passing on the data to the lower layers.

When receiving a telegram: each layer will process (=decode) its relevant information before passing the data on to the upper layers.

### The 2 types of communication

#### Connection-oriented communication:

The established connection constitutes a logical link between maximum two communication partners. Both partners regard this connection as a link that only exists between them (point-to-point [P2P] connection) and that can only be used by them.

The server (= the device wishing to establish a communication) must establish this connection according to some specified criteria (quality, cost, time delays, etc.) and uses resources to do so. In addition, a protocol or parameters, such as the data block length may be defined. When the connection is established, the actual data exchange (data transport) is executed according to the protocol.

The connection may be aborted at any time by any of the partners (disconnection). The resources used for the link are then released and made available to other functions.

Such communication is highly reliable, but is not very bandwidth effective (i.e., a lot of communication is needed to reach only one single communication partner).

When considering the OSI model, during this communication the Transport Layer as well as the Link Layer will generate messages confirming reception of data.

This type of communication can be compared to talking to one single person and checking after each question, whether he/she understood the question.

Use in KNX:

Downloading group addresses, parameters and/or application program from the PC (= ETS) into the KNX device, reading the device information.

#### Connectionless communication

This type of communication uses no connection set-up / disconnection rules. It is therefore not possible for the involved devices:

to agree upon a protocol or parameters

to fix criteria such as service quality, cost, etc.

As there is no connection set-up nor disconnection rule (no prior arrangement), the address must be sent together with the useful data with each data request. There is no confirmation of the data received and the device sending the information (=server) does not guarantee the correct order of the transmitted data blocks.

This kind of communication is not secure, but very bandwidth effective: there is no need to establish or break down a communication connection and with one single message, it is possible to reach all devices included in the installation. However, only one common acknowledgement of receipt is received. It can therefore not be ensured that all devices have correctly received the sent message.

When considering the OSI model, during this communication only the Link Layer will generate acknowledgements of receipt.

This type of communication can be compared to talking to a group of persons and continuing with the next question if at least one person confirms that he/she understood the message.

Use in KNX:

Normal operating mode of the KNX (telegrams with group addresses, multicast (one to many communication)

Broadcast communication (one to all communication)

### Protocol Data Units (PDUs)

This term is used to describe data as it moves from one layer of the OSI model to another. The Protocol Data Unit of an OSI layer constitutes

the data that is contributed by this Layer to the telegram when sent;

The data that is decoded by this Layer from the telegram when received;

# Principle functions of the Layers – Implementation in KNX

The underneath description focuses on the realization of KNX on the TP medium. For deviations to this for different media, please consult the relevant chapter in this documentation.

## Application Layer

The Application Layer is the link between the application program and the rest of the communication stack. It is amongst others responsible for the correct handling of the group object values of an application program.

Two cases can be distinguished in this context:

- Function when sending a telegram:

- The application program checks e.g., via the flags set in RAM, for possible user events

- One of the tasks of the Application Layer is to check for these user events

- A user event could be e.g., a rocker manipulation (or temperature measurement etc.) which will result in a GroupValueWrite telegram that will be sent on to the bus ( via the lower layers).

- Function when receiving a telegram:

- The Application Layer signals e.g., via the flags set in RAM, that data are being updated by the underlying communication stack;

- One of the routine tasks of the application program is to check for data that are being updated

- In case of e.g., a switch actuator: based on the updated data, the application program e.g., will open or close a relay.

Please note that the Application Layer is only aware of its specific data, e.g., the individual address of the sender of this data is unknown to the Application Layer.

## Presentation Layer (Not used in KNX)

This layer of the OSI model has the task of shielding Layer 7 and the application program from the duty of having to deal with different forms of representation of the transmitted data. It interprets and, if necessary, adjusts or converts the syntax of the message.

For example, a file containing the particulars of a group of persons must be transmitted from one bus device to another. The first device uses data records containing fields in the order "name, first name", whereas the second device uses the reverse order. In this case, it would be the task of the presentation layer to ensure that each device receives the data record in the correct order; layer 6 must therefore automatically switch the order of the data record fields as required.

With field buses in general, and thus also in KNX, the problem of converting the form of representation in the transmitted data does not arise. Hence, KNX devices do not need a Presentation Layer.

## Session Layer (Not used in KNX)

The principal task of Layer 5 according to OSI is to control the communication between two communication partners. This involves:

opening a dialog,

closing a dialog,

aborting or interrupting a dialog,

continuing a dialog at a later stage,

picking up a dialog at a defined, backdated point of time in the event of detected errors, so that a faulty data transmission can be repeated correctly.

As the messages exchanged in field buses are short (measured values), an explicit communication control is not required. Hence, KNX devices do not need a Session Layer.

## Transport Layer

It has two roles:

### Role 1: Actual transport of the data

4 different ‘transport’ data types can be distinguished:

- Data Packet without sequence number:

This is typically between sensors and actuators

This concerns runtime telegrams, such as communication between group objects (by GroupValueWrite telegrams)

- Data Packet with sequence number:

This is typically initiated by an interface, i.e., during device commissioning;

This concerns e.g., the writing of memory of devices (by a MemoryWrite telegram)

The sequence numbers serve the purpose of checking the commissioning completeness

- Control data without sequence number

This is typically initiated by an interface, i.e., in order to start and stop a device commission session

Two types of telegrams exist in this case:

T_Connect: to start a commissioning session

T_Disconnect: to end the commissioning session

Note: The established connection is automatically closed (T_Disconnect) by the sender under the following conditions:

- If a telegram cannot be transmitted within the set time span (Connection-oriented communication is monitored by means of timers);

- If neither T_ACK nor T_NAK is sent by the receiver

- If the sequence number sent by the sender does not have the value expected by the receiver (first the message will be repeated up to three times by the sender before closing the connection)

- Control data with sequence number

These are typically exchanged during a commissioning session, when e.g., writing memory of the destination device

The control data with sequence number are exchanged in 2 directions:

from interface to destination device and vice versa

i.e., per direction: a separate sequence number (counter) from 0…15 (4-bit) is used

Two types of telegrams exist in this case:

T_ACK: to indicate that the received telegram is/can be processed as expected

T_NAK: to indicate that the received telegram is NOT/can NOT be processed as expected. In this case, the message will be repeated up to three times by the sender. The repetition parameter is fixed in the system software.

### Role 2: Serving the Application Layer

Two different functions can be distinguished here:

- Function during commissioning

Received telegram: The Transport Layer forwards data to the Application Layer if the destination address of the telegram is the individual address of the device

Send telegram: The Application Layer forwards data to the Transport Layer in order to communicate with the initiator of the commissioning session, i.e., the interface

- Function during runtime

Received telegram: The Transport Layer consults the Association table in order to update the value of the destination group object(s). After this it informs the Application Layer about the update

Send telegram: The Application Layer informs the Transport Layer that the value of a group object is updated. After this it consults the Association table in order to put the correct group address (sending group address) in the telegram.

## Network Layer

The Network Layer functions according to the following principle:

- There shall only be one path between any set of communication partners

- There shall be a maximum of 6 hops between any set of communication partners

This means in other words that the Network Layer of any KNX device focuses on keeping track and/or processing of the so-called hop count, further abbreviated as HC. This HC is included in each telegram sent by the device.

The term 'hop' refers to the passing of any device having two segments such as a coupler or repeater.

### Device behavior

Interfaces

- have the possibility to send out telegrams with HC = 7 if clients use this value (ETS does not)

Regular devices (sensors and actuators):

send out telegrams with HC = 6 (maximum)

Couplers, at reception:

telegrams with 1 ≤ HC ≤ 7 shall be forwarded with HC-1

telegrams with HC = 0 shall be blocked

The routing mechanism works in the same way in media couplers between TP and RF, IP routers ... For more information, see chapter “Couplers” of the KNX Advanced course.

## Link Layer

The Link Layer handles the following tasks:

- Format: The Link Layer checks whether all received telegrams and all telegrams to be sent, are error-free.

- Content: The Link Layer parses all received telegrams and builds up all telegrams to be sent via an interface according to the KNX standardized 'Common External Message Interface' (cEMI).

In order to detect possible errors in telegrams, the Link Layer uses information from the Physical Layer:

- the parity bit of each individual octet (see 0)

- the check octet

By means of the parity bit, the Link Layer conducts per individual octet a so called horizontal parity check'. In case of KNX TP1 this is an even parity check: if the number of individual bits of an octet that are set to '1' is an even number, then the parity bit shall be set to '0'.

By means of the check octet, Link Layer conducts per telegram a so-called ‘vertical parity check'. In case of KNX TP1 this is a NOT XOR function: the value of the check octet shall be the result of this logical function applied upon all proceeding octets of the telegram.

Figure 5: Structure of the check octet

### Some examples

Table 1: Even vertical parity check (vertical parity) after each character and odd horizontal parity check (Checksum, horizontal parity) after each telegram

| Character | Octet |  |  |  |  |  |  |  |  | Even vertical parity bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 1 | 1 |  | 1 | 1 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 | 1 |  | 0 | 0 | 0 | 1 | 1 |
| 3 | 0 | 0 | 0 | 0 |  | 1 | 0 | 1 | 0 | 0 |
| 4 | 0 | 1 | 0 | 0 |  | 1 | 0 | 0 | 0 | 1 |
| 5 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 | 1 |
| 6 | 1 | 1 | 1 | 0 |  | 0 | 0 | 0 | 1 | 0 |
| 7 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |
| 8 | 1 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 1 |
| Odd horizontal parity bit | 0 | 1 | 1 | 1 |  | 0 | 0 | 0 | 0 |  |

There is a single-bit error in the 4th character. This error is detected by

the even vertical parity check, because the total number of ones in the data – plus the parity bit – is odd instead of even

whereas the exact position of the incorrect bit is recognized by the odd horizontal parity check (because the total number of ones in the data – plus the parity bit – is even instead of odd). This makes it possible to correct errors.

Table 2: Even vertical parity check (vertical parity) after each character and odd horizontal parity check (Checksum, horizontal parity) after each telegram

| Character | Octet |  |  |  |  |  |  |  |  | Even vertical parity bit |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 1 | 0 | 1 | 1 |  | 1 | 1 | 0 | 0 | 1 |
| 2 | 0 | 1 | 0 | 1 |  | 0 | 0 | 0 | 1 | 1 |
| 3 | 0 | 0 | 0 | 0 |  | 1 | 0 | 1 | 0 | 0 |
| 4 | 0 | 1 | 0 | 0 |  | 0 | 0 | 0 | 0 | 1 |
| 5 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 1 | 1 |
| 6 | 1 | 1 | 1 | 0 |  | 0 | 0 | 0 | 1 | 0 |
| 7 | 0 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 0 |
| 8 | 1 | 0 | 0 | 0 |  | 0 | 0 | 0 | 0 | 1 |
| Odd horizontal parity bit | 0 | 1 | 1 | 1 |  | 0 | 0 | 0 | 0 |

Bit error		 Bad parity

The 4th character contains a 2-bit error which remains undetected by the even vertical parity check but is recognized by the odd horizontal parity check. A correction is not possible here.

A telegram conversion takes place between Link Layer and Physical Layer. An important aspect of this conversion is the fact that when sending, the Link Layer on TP will add information that is used for error detection, e.g., start, stop, parity bit and check octet. The check octet is only used by the Link Layer of the receiver to check for possible errors (format). The check octet has no further significance (content) and is therefore not forwarded to the upper layers of the communication stack.

### Telegram structure according to cEMI

Figure 6: Bus side (TP1) vs cEMI side

The structure of KNX telegrams depends actually on from which “angle” one looks at a telegram. ETS records telegrams via a USB/TP or an IP/TP interface. Such interfaces have two sides: the BUS-side and the ETS-side.

An ETS monitor displays the telegram in an ETS-side telegram structure, an oscilloscope would display the telegram on the Bus-side in a different telegram structure.

When a KNX device sends a telegram on to the bus, this telegram will have a structure according to the KNX Physical Layer specifications of the medium used. The interface receiving that telegram will of course have the same structure on its BUS-side. The interface will internally however convert the telegram, i.e., the same telegram will have on its ETS-side a structure according to the so-called KNX cEMI specifications.

cEMI stands for “common External Message Interface” and is a protocol of which the responsibility is to ensure that messages sent on the BUS-side are correctly “converted” to external user applications (i.e., the ETS-side) and vice versa. This requires of course that both the KNX interface manufacturers as well as the ETS software abide by the same cEMI protocol.

Any further explanation related to the structure of KNX telegrams is according to the KNX cEMI specifications (= ETS bus monitor). This approach has the advantage that the telegram (and its structure) as recorded by the ETS monitor can be analyzed by the explanations given in this chapter.

Table 3: Basic frame structure for Link Layer Data messages (L_Data) according to cEMI

| 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Message Code | Additional Info Length | Additional information … | Control field 1 | Control field 2 | Source High | Source. Low | Dest. High | Dest. Low | NPDU |  |
| MC | AddIL |  | Ctrl1 | Ctrl2 | SAH | SAL | DAH | DAL | Information Length | TPCI/APCI & data |
| 1 octet | 1 octet | See below | 1 octet | 1 octet | 2 octets |  | 2 octets |  | 1 octet | Var. length |

The fields are defined as follows:

| Field | Meaning |
| --- | --- |
| MC | Message Code |
| AddIL | Additional Info Length |
| ---- | Additional information, in the case where AddIL is > 0 |
| Ctrl1 | Control field 1 |
| Ctrl2 | Control field 2 |
| SAH | Source address High |
| SAL | Source address Low |
| DAH | Destination address High |
| DAL | Destination address Low |
| NPDU | Network Protocol Data Unit |

#### 1 - Message Code

The Message Code indicates among other things whether the telegram comes from the bus or whether it was sent to the bus. It also gives information about the basic type of telegram.

- MC = 2E (hex): sent by the device to the connected client (interface connected to a client like ETS) to confirm that the message was correctly sent onto the bus, as requested by the client

- MC = 29 (hex): sent by the device to the connected client (interface connected to a client like ETS) to indicate that a message was received from the bus.

#### 2 - Additional Info Length + 3 - Additional Information

This field covers the AddIL octet + the Additional Information octets.

The Additional Information octets (3) are none existing if the AddIL octet is set to 0. This is always the case for TP and IP telegrams.

In case of the Additional Information Type “RF Medium Information”, the AddIL octet is set to 08h (8d). In this case the Additional Information octets contain among other things the RF KNX Serial number or the Domain Address.

#### 4 – Control field 1

The Control field 1 is one octet long and its data bits are encoded as follows:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FT | 0 | R | SB | P |  | A | E |

- FT: Frame type

  - 0 = extended frame (max. 263 octets). Extended frames are used for management, Radio Frequency and for secured frames

  - 1 = standard frame (max. 23 octets)

- R: Repeat flag

  - 0 = repeated telegram (in case of errors)

  - 1 = original telegram (not repeated)

- SB: System broadcast flag

  - 0 = system broadcast applied, i.e., in case of RF

  - 1 = 'regular' broadcast applied, i.e., in case of TP or IP

- P: Priority

  - 00 = system

  - 10 = alarm

  - 01 = high

  - 11 = low

- A: Acknowledgement flag

  - 0 = a LL_ACK is NOT requested for the telegram

  - 1 = a LL_ACK is requested for the telegram

- E: Error flag

  - 0 = the telegram contains NO errors

  - 1 = the telegram contains at least one error

#### 5 – Control field 2

The Control field 2 is one octet long and its data bits are encoded as follows:

| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| AT | HC |  |  | 0 | 0 | 0 | 0 |

- AT: address type

  - 0 = individual address (unicast)

  - 1 = group address (multicast) or 0/0/0 (broadcast)

- HC: hop count

  - see Network Layer §3.5

#### 6-7 – Source Address High + Source Address Low (SAH + SAL)

The source address covers the fields SAH + SAL. Both fields are one octet long.

These fields combined represent the Individual Address of the sending device, which is encoded as: A.L.D

| SAH |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| A: area |  |  |  | L: line |  |  |  |

| SAL |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| D: device address |  |  |  |  |  |  |  |

#### 8-9 – Destination Address High + Destination Address Low (DAH + DAL)

The destination address covers the fields DAH + DAL. Both fields are one octet long.

Both these fields combined represent either a group address, an individual address, or the broadcast address.

The 3-level group address structure is shown below:

| DAH |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| Main |  |  |  |  | Middle |  |  |

| DAL |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| Sub |  |  |  |  |  |  |  |

The most significant bit (D7) of Control field 2 needs to be considered for the encoding. The encoding is as follows:

| DAH + DAL | AT (Ctrl2:D7) | Communication type | ETS |
| --- | --- | --- | --- |
| All bits = 0 | 1 | Broadcast | 0/0/0 |
| At least 1 bit = 1 | 1 | Multicast | group address |
| At least 1 bit = 1 | 0 | Unicast | individual address |

#### 10 – NPDU

This field covers the L octet and the combination of TPCI, APCI and data. Also referred to as ‘Payload’.

NPDU = network protocol data unit

TPCI = Transport Layer protocol control information

APCI = Application Layer protocol control information

The L octet indicates the index of the last octet of the telegram, the index starts from the octet following the L octet, the value of the index starts from 0.

Based on the value of L, two categories of telegrams can be distinguished:

- L = 0: control data

- L > 0: data package

The TPCI is either 6 bits or 8 bits long.

The APCI is either 4 bits or 10 bits long.

Note: Only in the case of ‘UserMessage’ services (APCI = 1011) and ‘Escape’ services (APCI = 1111), the APCI is 10 bits.

##### L = 0: control data

If L = 0, then the NPDU consists entirely out of TPCI. There is no APCI or data.

The telegram contains either UCD or NCD.

UCD = unnumbered control data

NCD = numbered control data

The TPCI is in this context always 8 bits long. The most significant bit (D7) of the TPCI is in this context always set to 1.

| Octet: index 0 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | TPCI |
| UCD |  |  |  |  |  |  |  |  |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 0 | T_Connect |
| 1 | 0 | 0 | 0 | 0 | 0 | 0 | 1 | T_Disconnect |
| NCD |  |  |  |  |  |  |  |  |
| 1 | 1 | SeqNr |  |  |  | 1 | 0 | T_ACK |
| 1 | 1 | SeqNr |  |  |  | 1 | 1 | T_NAK |

Control data telegrams

The purpose of control data telegrams is to contribute to the controlling of any data exchange 'session' between two devices, typically between ETS (interface) and its destination device for commissioning.

4 control data telegrams can be distinguished:

| (T_Connect) | the source device requests the destination device to build up a unicast connection in order to exchange information |
| --- | --- |
| (T_Disconnect) | the source device requests the destination device to end their unicast connection because there is e.g., no more information to be exchanged |
| (T_ACK) | the destination device confirms the source device that it well 'understood' its last delivered telegram |
| (T_NAK) | the destination device confirms the source device that it did NOT well 'understood' its last delivered telegram |

Sequence Number

As mentioned in an earlier paragraph, the connection-oriented communication is monitored with sequence numbers ranging from 0 until 15 (4 bit value).

When device A sends a message to device B, it will start the sequence number at 0 and will increment it with each sent new message to reach 15. After that it starts at 0 again.

The same thing happens when device B sends messages to device A, in other words, both processes run separately from each other.

T_ACK vs. LL_ACK

T_ACK shall not be confused with LL_ACK:

- LL_ACK:

the destination device confirms the source device that it has well received its latest delivered telegram.

it is possible that more than one device sends such confirmation for the same delivered telegram, e.g., in case of GroupValueWrite (multicast)

such a telegram is not a complete telegram, i.e., it contains only one octet.

- T_ACK:

this is as explained above all about understanding telegrams.

such confirmation can only be sent by one device, i.e., the destination device (unicast)

such a telegram is always combined with an LL_ACK: the destination device can only understand telegrams that were well received in the first place.

##### L > 0: data package

If L > 0, then the NPDU consists of TPCI, APCI and data.

The telegram contains either UDP or NDP.

UDP = unnumbered data package

NDP = numbered data package

The TPCI is in this context always 6 bits long. The most significant bit (D7) of the TPCI is in this context always set to 0. The next bit determines whether it is a numbered data package (NDP) or an unnumbered data package (UDP).

| Octet: index 0 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| UDP |  |  |  |  |  |  |  |
| 0 | 0 | 0 | 0 | 0 | 0 | APCI |  |
| NDP |  |  |  |  |  |  |  |
| 0 | 1 | SeqNr |  |  |  | APCI |  |

Telegrams can either be sent as UDP or NDP: see example 2 and 4 below.

Sequence Numbers

In case of Unnumbered (D6 = 0 of octet index 0), the next 4 bits have no meaning (always set to 0)

In case of Numbered (D6 = 1 of octet index 0), the next 4 bits indicate the sequence number.

The (sequence) numbering depends on the direction:

- either from communication partner A to communication partner B

- or vice versa

The sequence number starts (per direction) from 0 till 15, after 15 it elapses back to 0.

Four possible combinations can in this context be distinguished:

|  | TPCI: D6 = 0 | TPCI: D6 = 1 |
| --- | --- | --- |
| APCI length = 4 bits | See example 1 below | See example 3 below |
| APCI length = 10 bits | See example 2 below | See example 4 below |

Example 1: GroupValueWrite telegram for a dimming control object

| Octet: index 0 |  |  |  |  |  |  |  | Octet: index 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| 0 | 0 | 0 | 0 | 0 | 0 | APCI |  |  |  | data |  |  |  |  |  |

Number of data fields for a GroupValueWrite telegram = 1

location of the data field: octet index 1, bits D5-D0

Possible interpretation: Any DPT fitting within 6 bits, e.g.:

dimming control command, e.g., dim up in 16 steps.

Switch telegram

…

Example 2: PropertyValueRead telegram (when sent connectionless)

| Octet: index 0 |  |  |  |  |  |  |  | Octet: index 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| 0 | 0 | 0 | 0 | 0 | 0 | APCI |  |  |  |  |  |  |  |  |  |

number of data fields for a PropertyValueRead telegram = 4

location of the data fields:

data field 1 = octet index 2

data field 2 = octet index 3

data field 3 + 4 = octet index 4 + 5

interpretation of the fields:

data field 1 = the device object ID

data field 2 = the property ID

data field 3 + 4 = count + start index

Example 3: MemoryResponse telegram

| Octet: index 0 |  |  |  |  |  |  |  | Octet: index 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| 0 | 1 | SeqNr |  |  |  | APCI |  |  |  | data |  |  |  |  |  |

number of data fields of a MemoryResponse telegram = 3

location of the data fields:

data field 1 = octet index 1, bits D5-D0

data field 2 = octet index 2 and octet index 3

data field 3 = octet index 4 until octet defined in index L

interpretation in the case when e.g., a MemoryRead took place:

data field 1: the number of octets read from memory (max. 64 octets)

data field 2: the memory location within the device

data field 3: the actual octets that are read

Example 4: PropertyValueRead telegram (when sent connection-oriented)

| Octet: index 0 |  |  |  |  |  |  |  | Octet: index 1 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 | D7 | D6 | D5 | D4 | D3 | D2 | D1 | D0 |
| 0 | 1 | SeqNr |  |  |  | APCI |  |  |  |  |  |  |  |  |  |

number of data fields of a PropertyValueRead telegram = 4

location of the data fields:

data field 1 = octet index 2

data field 2 = octet index 3

data field 3 + 4 = octet index 4 + 5

interpretation of the fields:

data field 1 = the device object ID

data field 2 = the property ID

data field 3 + 4 = count + start index

L vs. data length

The L octet does not necessarily indicate the length of the data.

The data length correlates with the target (type), which correlates with the APCI.
3 target types can (in this context) be distinguished.

Object

device memory

property

Example: a GroupValueWrite telegram with L = 1

Destination address = Group address => target type is object

This means that the length of the data in this example is minimum 1 bit and maximum 6 bit long

e.g., 1-bit: the destination is e.g., an On/Off object.

e.g., 4-bit: the destination is e.g., a dimming control object.

Any data larger than 6 bits will result in a telegram length of 2.

Data fields

The data as encapsulated in a telegram is in most cases contained within one field but can also be divided over several data fields. The number of data fields and their locations within the telegram is defined by the APCI. The interpretation of the data (fields) is defined by the APCI.

##### The standard  APCI Codes  and their meaning

The task of the Application Layer is to:

manage the values of the group objects on behalf of the application program

process group telegrams as well as carry out management functions.

In run-time communication (i.e., after configuration), the typical APCIs used are:

| APCI | Name |
| --- | --- |
| 0000 | GroupValueRead |
| 0001 | GroupValueResponse |
| 0010 | GroupValueWrite |

These APCIs are used for multicast communication, i.e., the destination addresses of these telegrams are group addresses and useful data is coded according to the KNX Interworking Data types.

Configuration of bus devices is ensured via management functions, which are typically stored in the ROM of the bus device. For such management functions, typically connection-oriented (point-to-point) communication or broadcast (group address 0/0/0) is used. Please note that a user may not and cannot set the group address 0/0/0 in ETS itself.

The remaining standard APCIs are (also in ETS):

| APCI | Name |
| --- | --- |
| 0011 | PhysAddrWrite |
| 0100 | PhysAddrRead |
| 0101 | PhysAddrResponse |
| 0110 | AdcRead |
| 0111 | AdcResponse |
| 1000 | MemoryRead |
| 1001 | MemoryResponse |
| 1010 | MemoryWrite |
| 1011 | UserMessage |
| 1100 | DeviceDescriptorRead |
| 1101 | DeviceDescriptorResponse |
| 1110 | Restart |
| 1111 | Escape |

Some other specific APCIs are defined to:

Configure line and backbone couplers

Attribute keys for (memory) access protection

Grant authorization to access memory

Handling of the domain address in case of RF devices

Reading and writing of the individual address via the KNX serial number of a bus device

Addressing (properties of) KNX interface objects of a remote device (interface objects are explained in chapter Application program of Tutor course)

##### Detailed description of some APCIs

| APCI | APCI Coding & Explanation |
| --- | --- |
| GroupValueRead | 00; 00; xxxxxx. Explanation: xxxxxx: no significance (typically 0b) Reads the value of a group object(s) linked to a particular group address used as destination address in the message with this APCI |
| GroupValueResponse | - telegram format for useful data of up to 6 bits: 00; 01; dddddd. - telegram format for useful data larger than 6 bits: 00; 01; xxxxxx; dddddddd; . . .; dddddddd. Explanation: xxxxxx: no significance (typically 0b) d…….d: data Reply to a GroupValueRead request with the value of the group object(s) linked to the group address used in the read request (in BCU1 = value write for receiver!). The destination address used in a GroupValueResponse message is the sending group address attributed to the different group objects concerned. |

| GroupValueWrite | - telegram format for useful data of up to 6 bits: 00; 10; dddddd. - telegram format for useful data larger than 6 bits: 00; 10; xxxxxx; dddddddd; . . .; dddddddd. Explanation: xxxxxx: no significance (typically 0b) d…….d: data Write the value of a group object linked to a particular group address. The content of a group object is transmitted onto the bus, together with the sending group address assigned, e.g., when a push button sensor is operated. The content of a group object is overwritten if the group address received is assigned to the respective group object (e.g., relay switches on or off). |
| --- | --- |
| PhysAddressWrite | 00; 11; xxxxxx; BBBBLLLL; TTTTTTTT. Explanation: xxxxxx: no significance (typically 0b) BBBBLLLL TTTTTTTT: individual address to be assigned Writes the indicated individual address into the device in programming mode (the programming LED of this device is ON). The destination address in the telegram is 0/0/0 (broadcast). |
| PhysAddressRead | 01; 00; xxxxxx. Explanation: xxxxxx: no significance (typically 0b) Reads the individual address of all those bus devices within a KNX installation that are currently in programming mode (the programming LEDs of these devices light up). The destination address in the telegram is 0/0/0 (broadcast). |
| PhysAddressResponse | 01; 01; xxxxxx. Explanation: xxxxxx: no significance (typically 0b) Response to an PhysAddressRead of all those bus devices of a KNX installation that are currently in the programming mode (their programming LED lights up). The destination address in the telegram is 0/0/0 (broadcast). |
| AdcRead | 01; 10; aaaaaaa; cccccccc. Explanation: aaaaaaa: number of the A to D channel to be read cccccccc: number of A to D conversions to be carried out and added up Requests the addressed bus device to carry out a number of Analogue to Digital conversions, calculate the sum and return the result, of the indicated A to D channel. |
| AdcResponse | 01; 11; aaaaaa; cccccccc; hhhhhhhh; LLLLLLLL. Explanation: aaaaaa: number of the A to D channel read cccccccc: number of A to D conversions carried out and summed hhhhhhhh: high byte of the result LLLLLLLL: low byte of the result Response to AdcRead containing the requested data. |
| MemoryRead | 10; 00; LLLLLL; aaaaaaaa; bbbbbbbb. Explanation: LLLLLL: length of the memory area to be read (bytes) aaaaaaaa: starting address of the memory area to be read (high byte) bbbbbbbb: starting address of the memory area to be read (low byte) Requests the addressed bus device to transmit its memory contents, starting at the specified address. |
| MemoryResponse | 10; 01; LLLLLL; aaaaaaaa; bbbbbbbb; dddddddd; ...; dddddddd. Explanation: LLLLLL: length of the memory area read (bytes) aaaaaaaa: starting address of the memory area read (high byte) bbbbbbbb: starting address of the memory area read (low byte) d...d: contents of the memory area read Response to MemoryRead containing the requested data. If part of the requested data lies in a protected memory area or does not exist, then the bus device returns the message L...L = 0 and no data at all. |
| MemoryWrite | 10; 10; LLLLLL; aaaaaaaa; bbbbbbbb; dddddddd; . . .; dddddddd. Explanation: LLLLLL: number of bytes to be written aaaaaaaa: destination address of the 1st byte to be written (high byte) bbbbbbbb: destination address of the 1st byte to be written (low byte) d...d: data to be written Writes a continuous data block into the memory of the addressed bus device, starting at the specified address. If part of the requested data lies in a protected memory area or does not exist, then the bus device returns a Memory Response message L...L = 0 and no data (indicating that no data was written). When the mode called ‘Verify’ is activated, bus devices automatically emit a MemoryResponse immediately after the write instruction. |
| User Message | 10; 11; dddddd; . . .; dddddddd. Explanation: d...d: data Initiates the exchange of data between two microcontrollers in the bus device (this is rather an exception in current devices). |
| DeviceDescriptorRead | 11; 00; xxxxxx. Explanation: xxxxxx: no significance (typically 0b) Requests the addressed bus device to return information on its supported system profile by means of its mask type and mask version. |
| DeviceDescriptorResponse | 11; 01; xxxxxx; mmmmmmmm; vvvvuuuu. Explanation: xxxxxx: no significance (typically 0b) mmmmmmmm: mask type (first nibble indicates medium, second nibble indicates main profile) vvvvuuuu: mask version (first nibble main version, second nibble subversion) Response to DeviceDescriptorRead containing the requested data. For more information on the existing system profiles, see appropriate paragraphs in Basic Course, Part ‘Bus devices’ |
| Restart | 11; 10; xxxxxx. Explanation: xxxxxx: no significance (typically 0b) Resets the addressed bus device (= complete restart including operating system and application program). Whether or not the application module, too, is reset, depends on the respective bus device to which the message was sent. |
| Escape | 11; 11; eeeeee. Explanation: e...e: extended APCI bits In case the APCI has the value “1111b”, the exact meaning of the APCI is indicated in the six bits following the Escape APCI. |
| AuthorizeRequest AuthorizeResponse (in case of eeeeee = 010 001, respectively 010 010) | These services allow accessing a bus device with memory access-protection. 16 different access levels are possible. A 32-bit number (or 4 bytes) is required to be granted access to memory. If no access protection is used, the number remains at FFFF FFFFh and all the access levels are enabled. The process is started by an M_ AuthorizeRequest message that contains the number. The device that receives the message compares the number with its table and enables the corresponding access levels. If the number is not in the table, the device disables all memory access. The bus device replies with an M_ AuthorizeResponse; this reply contains the information about to which level access has been granted. |
| SetKeyRequest SetKeyResponse (in case of eeeeee = 010 011 respectively 010 100) | These services allow to write keys for the various access levels to bus devices with memory access protection (16 levels are possible, 0 is the highest level and 15 is the lowest). A code is set by an M_SetKeyRequest, indicating the key for the respective access level. The receiving device tests whether the current access level is at least as high as the one that is to receive the indicated key. If that is the case, the code is set and an M_SetKeyResponse is returned together with the access level. If the test proves negative, an M_SetKeyResponse is returned together indicating the error code FFh. |

#### Examples

- 2E 00 BC E0 11 FA 00 01 01 00 81

| 1 | 2E |  | Sent by a device |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11FA | 0001 0001 1111 1010 | D0-D7 1111 1010 = Device address (250) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.250 |
| 8-9 | 0001 | 0000 0000 0000 0001 | D7 of Control field 2 = 1 Group address At least one bit = 1 Group address (multicast) D0-D7 = 1 D0-D2 = 0 D3-D7 = 0 Group address = 0/0/1 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 00 | 0000 0000 | 00 TPCI = UDP 0000 no meaning 00 = APCI |
|  | 81 | 1000 0001 | 10 = APCI. The APCI is thus = 0010, meaning a GroupValueWrite telegram Data: 000001 Explanation: GroupValueWrite telegram with value 1 (e.g., switch object) on group address 0/0/1 |

- 2E 00 B0 60 11 FA 11 01 00 80

| 1 | 2E |  | Sent by a device |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11FA | 0001 0001 1111 1010 | D0-D7 1111 1010 = Device address (250) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.250 |
| 8-9 | 1101 | 0001 0001 0000 0001 | D7 of Control field 2 = 0 Individual address At least one bit = 1 Individual address (unicast) D0-D7 = 1 D0-D3 = 1 D0-D3 = 1 Individual address = 1.1.1 |
| 10 | 00 | 0000 0000 | L = 0 control data |
|  | 80 | 1000 0000 | 1000 0000 TPCI (NPDU = completely TPCI) of type UCD Explanation: T_Connect message from device with address 1.1.250 to a device with address 1.1.1 |

- 2E 00 B0 60 11 FA 11 01 03 66 04 01 04

| 1 | 2E |  | Sent by a device |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11FA | 0001 0001 1111 1010 | D0-D7 1111 1010 = Device address (250) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.250 |
| 8-9 | 1101 | 0001 0001 0000 0001 | D7 of Control field 2 = 0 individual address At least one bit = 1 Individual address (unicast) D0-D7 = 1 D0-D3 = 1 D0-D3 = 1 Individual address = 1.1.1 |
| 10 | 03 | 0000 0011 | L > 0 data package |
|  | 66 | 0110 0110 | 01 TPCI = NDP 1001 sequence number = 9 10 = APCI |
|  | 04 | 0000 0100 | 00 = APCI. The APCI is thus = 1000, meaning a MemoryRead telegram Data = 00 0100 |
|  | 0104 | 0000 0001 0000 0100 | 0000 0001 0000 0100 = data Explanation: MemoryRead – initiated by device 1.1.250 towards device 1.1.1 – 4 octets starting from memory address = $0104 |

- 29 00 BC 60 11 01 11 FA 07 5E 44 01 04 FA 01 02 01

| 1 | 29 |  | From bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 1101 | 0001 0001 0000 0001 | D0-D7 0000 0001 = Device address (1) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.1 |
| 8-9 | 11FA | 0001 0001 1111 1010 | D7 of Control field 2 = 0 individual address At least one bit = 1 Individual address (unicast) D0-D7 = 250 D0-D3 = 1 D0-D3 = 1 Individual address = 1.1.250 |
| 10 | 07 | 0000 0111 | L > 0 data package |
|  | 5E | 0101 1110 | 01 TPCI = NDP 0111 sequence number = 7 10 = APCI |
|  | 44 | 0100 0100 | 01 = APCI. The APCI is thus = 1001, meaning a MemoryResponse telegram Data = 00 0100 |
|  | 0104FA010201 | 0000 0001 0000 0100 1111 1010 0000 0001 0000 0010 0000 0001 | 0000 0001 0000 0100 1111 1010 0000 0001 0000 0010 0000 0001 = data Explanation: MemoryResponse telegram – from device with address 1.1.1 towards initiating device 1.1.250, 4 octets read from memory address = $0104, information $FA010201 |

- 2E 00 B0 60 11 FA 11 01 00 CE

| 1 | 2E |  | Sent by a device |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11FA | 0001 0001 1111 1010 | D0-D7 1111 1010 = Device address (250) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.250 |
| 8-9 | 1101 | 0001 0001 0000 0001 | D7 of Control field 2 = 0 individual address At least one bit = 1 Individual address (unicast) D0-D7 = 1 D0-D3 = 1 D0-D3 = 1 Individual address = 1.1.1 |
| 10 | 00 | 0000 0000 | L = 0 control data |
|  | CE | 1100 1110 | 1100 1110 TPCI (NPDU = completely TPCI) of type NCD with sequence number 00 11 (=3) Explanation: T_ACK message from device 1.1.250 towards device 1.1.1, indicating that the previous message in a unicast communication has been understood. |

- 2E 00 B0 60 11 FA 11 01 01 63 80

| 1 | 2E |  | Sent by a device |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11FA | 0001 0001 1111 1010 | D0-D7 1111 1010 = Device address (250) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.250 |
| 8-9 | 1101 | 0001 0001 0000 0001 | D7 of Control field 2 = 0 individual address At least one bit = 1 Individual address (unicast) D0-D7 = 1 D0-D3 = 1 D0-D3 = 1 Individual address = 1.1.1 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 63 | 0110 0011 | 01: Numbered data package 1000: Sequence number = 8 11 = APCI |
|  | 80 | 1000 0000 | 10 = APCI The APCI is thus = 1110, meaning a Restart telegram 00 0000: data Explanation: the device with address 1.1.250 requests the device 1.1.1 to perform a restart, i.e., rebooting system and application software |

- 2E 00 BC E0 11 FA 01 03 01 00 00

| 1 | 2E |  | Sent by a device |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11FA | 0001 0001 1111 1010 | D0-D7 1111 1010 = Device address (250) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.250 |
| 8-9 | 0103 | 0000 0001 0000 0011 | D7 of Control field 2 = 1 group address At least one bit = 1 group address (multicast) D0-D7 = 3 D0-D2 = 1 D3-D7 = 0 Group address = 0/1/3 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 00 | 0000 0000 | 00: Unnumbered data package 0000 = don’t care 00 = APCI |
|  | 00 | 0000 0000 | 00 = APCI The APCI is thus = 0000 which means a GroupValueRead telegram on Group address 0/1/3 00 0000: data Explanation: device with address 1.1.250 sends on group address 0/1/3 a group value read message to the group object(s) listening to this group address. |

- 29 00 BC E0 11 01 01 03 02 00 40 CC

| 1 | 29 |  | From bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority telegram D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 1101 | 0001 0001 0000 0001 | D0-D7 0000 0001 = Device address (1) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.1 |
| 8-9 | 0103 | 0000 0001 0000 0011 | D7 of Control field 2 = 1 group address At least one bit = 1 group address (multicast) D0-D7 = 3 D0-D2 = 1 D3-D7 = 0 Group address = 0/1/3 |
| 10 | 02 | 0000 0010 | L > 0 data package |
|  | 00 | 0000 0000 | 00: Unnumbered data package 0000 = don’t care 00 = APCI |
|  | 40 | 0100 0000 | 01 = APCI The APCI is thus = 0001, meaning a GroupValueResponse telegram from Group address 0/1/3 00 0000: data Explanation: the device with address 1.1.1 sends on via its sending group address 0/1/3 the value of the connected group object, see below |
|  | CC | 1100 1100 | 1100 1100: value 204 (when interpreted as an unsigned integer value) |

- 2E 00 B0 E0 11 FA 00 00 01 01 00

| 1 | 29 |  | Sent by Device |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11FA | 0001 0001 1111 1010 | D0-D7 1111 1010 = Device address (250) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.1 |
| 8-9 | 0000 | 0000 0000 0000 0000 | D7 of Control field 2 = 1 group address All bits = 0 0/0/0 (Broadcast) D0-D7 = 0 D0-D2 = 0 D3-D7 = 0 Broadcast = 0/0/0 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 01 | 0000 0001 | 00: Unnumbered data package 0000 = don’t care 01 = APCI |
|  | 00 | 0000 0000 | 00 = APCI The APCI is thus = 0100, meaning a PhysAddrRead 00 0000 = data Explanation: the device with address 1.1.250 requests from all devices on the bus that are in programming mode to send back their individual addresses. |

- 29 00 BC E0 11 01 00 00 01 01 40

| 1 | 29 |  | From bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - Does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 1101 | 0001 0001 0000 0001 | D0-D7 0000 0001 = Device address (1) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.1 |
| 8-9 | 0000 | 0000 0000 0000 0000 | D7 of Control field 2 = 1 group address All bits = 0 0/0/0 (Broadcast) D0-D7 = 0 D0-D2 = 0 D3-D7 = 0 Broadcast = 0/0/0 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 01 | 0000 0001 | 00: Unnumbered data package 0000 = don’t care 01 = APCI |
|  | 40 | 0100 0000 | 01 = APCI The APCI is thus = 0101, meaning a PhysAddressResponse 00 0000 = data Response = Individual address 1.1.1 Explanation: the device with address 1.1.1 replies to the PhysAddressRead with its attributed individual address. |

### TL Session example: Downloading a KNX Virtual application program by means of ETS

| # | Source Address | Destination Address | Type | Info | Comments |
| --- | --- | --- | --- | --- | --- |
| 1 | 1.1.255 | 1.1.1 | T_Connect |  | Start TL session |
| 2 | 1.1.255 | 1.1.1 | DeviceDescriptorRead (S=0) | DescriptorType=0 | Get the mask version |
| 3 | 1.1.1 | 1.1.255 | T_ACK (S=0) |  |  |
| 4 | 1.1.1 | 1.1.255 | DeviceDescriptorResponse (S=0) | DescriptorType=0, DescriptorData=07 B0 | ($07B0 = System B - TP) |
| 5 | 1.1.255 | 1.1.1 | T_ACK (S=0) |  |  |
| 6 | 1.1.255 | 1.1.1 | PropertyValueRead (S=1) | ObjectIndex=0, PropertyId=56, Count=1, StartIndex=1 | Get maximum APDU length |
| 7 | 1.1.1 | 1.1.255 | T_ACK (S=1) |  |  |
| 8 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=1) | ObjectIndex=0, PropertyId=56, Count=1, StartIndex=1, Data=00 42 | Max. APDU length is 66 octets (Device indicates that it can handle 66 octets of payload (NPDU) per telegram) |
| 9 | 1.1.255 | 1.1.1 | T_ACK (S=1) |  |  |
| 10 | 1.1.255 | 1.1.1 | AuthorizeRequest (S=2) | 00 FF FF FF | Request if for authorization level ‘00’ no BCU key is required 00 = level of authorization (always ‘0’) |
| 11 | 1.1.1 | 1.1.255 | T_ACK (S=2) |  |  |
| 12 | 1.1.1 | 1.1.255 | AuthorizeResponse (S=2) | 00 | (OK) Confirmed. There is no BCU key for level 00 |
| 13 | 1.1.255 | 1.1.1 | T_ACK (S=2) |  |  |
| 14 | 1.1.255 | 1.1.1 | PropertyDescRead (S=3) | ObjectIndex=2, PropertyId=23, PropertyIndex=0 | Get Association table format |
| 15 | 1.1.1 | 1.1.255 | T_ACK (S=3) |  |  |
| 16 | 1.1.1 | 1.1.255 | PropertyDescResponse (S=3) | ObjectIndex=2, PropertyId=23, PropertyIndex=0, DataType=20 (PDT_GENERIC_04), MaxSize=200 |  |
| 17 | 1.1.255 | 1.1.1 | T_ACK (S=3) |  |  |
| 18 | 1.1.255 | 1.1.1 | PropertyValueRead (S=4) | ObjectIndex=0, PropertyId=11, Count=1, StartIndex=1 | Get serial number of device |
| 19 | 1.1.1 | 1.1.255 | T_ACK (S=4) |  |  |
| 20 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=4) | ObjectIndex=0, PropertyId=11, Count=1, StartIndex=1, Data=00 FA 00 00 23 04 |  |
| 21 | 1.1.255 | 1.1.1 | T_ACK (S=4) |  |  |
| 22 | 1.1.255 | 1.1.1 | PropertyValueRead (S=5) | ObjectIndex=0, PropertyId=12, Count=1, StartIndex=1 | Get manufacturer ID |
| 23 | 1.1.1 | 1.1.255 | T_ACK (S=5) |  |  |
| 24 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=5) | ObjectIndex=0, PropertyId=12, Count=1, StartIndex=1, Data=00 FA | ($00FA = KNX Association) Virtual KNX Association product |
| 25 | 1.1.255 | 1.1.1 | T_ACK (S=5) |  |  |
| 26 | 1.1.255 | 1.1.1 | PropertyValueRead (S=6) | ObjectIndex=0, PropertyId=25, Count=1, StartIndex=1 | Get device version |
| 27 | 1.1.1 | 1.1.255 | T_ACK (S=6) |  |  |
| 28 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=6) | ObjectIndex=0, PropertyId=25, Count=1, StartIndex=1, Data=48 00 |  |
| 29 | 1.1.255 | 1.1.1 | T_ACK (S=6) |  |  |
| 30 | 1.1.255 | 1.1.1 | PropertyValueRead (S=7) | ObjectIndex=0, PropertyId=78, Count=1, StartIndex=1 | Get hardware type |
| 31 | 1.1.1 | 1.1.255 | T_ACK (S=7) |  |  |
| 32 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=7) | ObjectIndex=0, PropertyId=78, Count=1, StartIndex=1, Data=00 00 00 00 00 54 |  |
| 33 | 1.1.255 | 1.1.1 | T_ACK (S=7) |  |  |
| 34 | 1.1.255 | 1.1.1 | PropertyValueRead (S=8) | ObjectIndex=0, PropertyId=15, Count=1, StartIndex=1 | Get order information |
| 35 | 1.1.1 | 1.1.255 | T_ACK (S=8) |  |  |
| 36 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=8) | ObjectIndex=0, PropertyId=15, Count=1, StartIndex=1, Data=16 75 C1 08 20 02 45 20 81 00 |  |
| 37 | 1.1.255 | 1.1.1 | T_ACK (S=8) |  |  |
| 38 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=9) | ObjectIndex=5, PropertyId=5, Count=1, StartIndex=1, Data=04 00 00 00 00 00 00 00 00 00 | Unload application program 2 (It will first unload the application program 2 if there is one) |
| 39 | 1.1.1 | 1.1.255 | T_ACK (S=9) |  |  |
| 40 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=9) | ObjectIndex=5, PropertyId=5, Count=1, StartIndex=1, Data=$00 | (OK) |
| 41 | 1.1.255 | 1.1.1 | T_ACK (S=9) |  |  |
| 42 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=10) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=04 00 00 00 00 00 00 00 00 00 | Unload application program (Then it will unload the application program if there is one) |
| 43 | 1.1.1 | 1.1.255 | T_ACK (S=10) |  |  |
| 44 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=10) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=$00 | (OK) |
| 45 | 1.1.255 | 1.1.1 | T_ACK (S=10) |  |  |
| 46 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=11) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=04 00 00 00 00 00 00 00 00 00 | Unload Object table |
| 47 | 1.1.1 | 1.1.255 | T_ACK (S=11) |  |  |
| 48 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=11) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=$00 | (OK) |
| 49 | 1.1.255 | 1.1.1 | T_ACK (S=11) |  |  |
| 50 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=12) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=04 00 00 00 00 00 00 00 00 00 | Unload Association table |
| 51 | 1.1.1 | 1.1.255 | T_ACK (S=12) |  |  |
| 52 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=12) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=$00 | (OK) |
| 53 | 1.1.255 | 1.1.1 | T_ACK (S=12) |  |  |
| 54 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=13) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=04 00 00 00 00 00 00 00 00 00 | Unload Address table |
| 55 | 1.1.1 | 1.1.255 | T_ACK (S=13) |  |  |
| 56 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=13) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=$00 | (OK) |
| 57 | 1.1.255 | 1.1.1 | T_ACK (S=13) |  |  |
| 58 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=14) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=01 00 00 00 00 00 00 00 00 00 | Prepare loading application program |
| 59 | 1.1.1 | 1.1.255 | T_ACK (S=14) |  |  |
| 60 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=14) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) 02 refers to ‘Confirmation of device for downloading the application program.’ |
| 61 | 1.1.255 | 1.1.1 | T_ACK (S=14) |  |  |
| 62 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=15) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=03 0B 00 00 01 00 00 00 00 00 | Allocate memory for loading application program |
| 63 | 1.1.1 | 1.1.255 | T_ACK (S=15) |  |  |
| 64 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=15) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) 02 refers to ‘Confirmation of device for allocating the required memory’ |
| 65 | 1.1.255 | 1.1.1 | T_ACK (S=15) |  |  |
| 66 | 1.1.255 | 1.1.1 | PropertyValueRead (S=0) | ObjectIndex=4, PropertyId=13, Count=1, StartIndex=1 | Get application program version (Sequence number starts again with 0) |
| 67 | 1.1.1 | 1.1.255 | T_ACK (S=0) |  |  |
| 68 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=0) | ObjectIndex=4, PropertyId=13, Count=1, StartIndex=1, Data=00 FA 02 04 23 |  |
| 69 | 1.1.255 | 1.1.1 | T_ACK (S=0) |  |  |
| 70 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=1) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=01 00 00 00 00 00 00 00 00 00 | Prepare loading Object table |
| 71 | 1.1.1 | 1.1.255 | T_ACK (S=1) |  |  |
| 72 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=1) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) |
| 73 | 1.1.255 | 1.1.1 | T_ACK (S=1) |  |  |
| 74 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=2) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=03 0B 00 00 00 90 00 00 00 00 | Allocate memory for loading Object table |
| 75 | 1.1.1 | 1.1.255 | T_ACK (S=2) |  |  |
| 76 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=2) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) |
| 77 | 1.1.255 | 1.1.1 | T_ACK (S=2) |  |  |
| 78 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=3) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=01 00 00 00 00 00 00 00 00 00 | Prepare loading Address table |
| 79 | 1.1.1 | 1.1.255 | T_ACK (S=3) |  |  |
| 80 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=3) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) |
| 81 | 1.1.255 | 1.1.1 | T_ACK (S=3) |  |  |
| 82 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=4) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=03 0B 00 00 00 0C 00 00 00 00 | Allocate memory for loading Address table |
| 83 | 1.1.1 | 1.1.255 | T_ACK (S=4) |  |  |
| 84 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=4) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) |
| 85 | 1.1.255 | 1.1.1 | T_ACK (S=4) |  |  |
| 86 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=5) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=01 00 00 00 00 00 00 00 00 00 | Prepare loading Association table |
| 87 | 1.1.1 | 1.1.255 | T_ACK (S=5) |  |  |
| 88 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=5) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) |
| 89 | 1.1.255 | 1.1.1 | T_ACK (S=5) |  |  |
| 90 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=6) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=03 0B 00 00 00 16 00 00 00 00 | Allocate memory for loading Association table |
| 91 | 1.1.1 | 1.1.255 | T_ACK (S=6) |  |  |
| 92 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=6) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=$02 | (OK) |
| 93 | 1.1.255 | 1.1.1 | T_ACK (S=6) |  |  |
| 94 | 1.1.255 | 1.1.1 | PropertyValueRead (S=7) | ObjectIndex=4, PropertyId=7, Count=1, StartIndex=1 | Get application program start address |
| 95 | 1.1.1 | 1.1.255 | T_ACK (S=7) |  |  |
| 96 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=7) | ObjectIndex=4, PropertyId=7, Count=1, StartIndex=1, Data=00 00 60 00 | (Start address = $6000) |
| 97 | 1.1.255 | 1.1.1 | T_ACK (S=7) |  |  |
| 98 | 1.1.255 | 1.1.1 | PropertyValueRead (S=8) | ObjectIndex=0, PropertyId=14, Count=1, StartIndex=1 | Get device status |
| 99 | 1.1.1 | 1.1.255 | T_ACK (S=8) |  |  |
| 100 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=8) | ObjectIndex=0, PropertyId=14, Count=1, StartIndex=1, Data=$04 | (OK) |
| 101 | 1.1.255 | 1.1.1 | T_ACK (S=8) |  |  |
| 102 | 1.1.255 | 1.1.1 | MemoryWrite (S=9) | Count=63, Address=$6000, Data=10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 20 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 30 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF | Write (As the application program contains more than 66 octets, ETS has to write it in several chunks). The first chunk starts as of address 6000h |
| 103 | 1.1.1 | 1.1.255 | T_ACK (S=9) |  |  |
| 104 | 1.1.1 | 1.1.255 | MemoryResponse (S=9) | Count=63, Address=$6000, Data=10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 20 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 30 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF | (OK) |
| 105 | 1.1.255 | 1.1.1 | T_ACK (S=9) |  |  |
| 106 | 1.1.255 | 1.1.1 | MemoryWrite (S=10) | Count=63, Address=$603F, Data=FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF | Write (The second chunk starts as of address 603Fh) |
| 107 | 1.1.1 | 1.1.255 | T_ACK (S=10) |  |  |
| 108 | 1.1.1 | 1.1.255 | MemoryResponse (S=10) | Count=63, Address=$603F, Data=FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF FF FF 10 01 02 05 FF FF FF FF FF FF FF FF FF FF | (OK) |
| 109 | 1.1.255 | 1.1.1 | T_ACK (S=10) |  |  |
| 110 | 1.1.255 | 1.1.1 | MemoryWrite (S=11) | Count=63, Address=$607E, Data=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Write (Chunk 3) |
| 111 | 1.1.1 | 1.1.255 | T_ACK (S=11) |  |  |
| 112 | 1.1.1 | 1.1.255 | MemoryResponse (S=11) | Count=63, Address=$607E, Data=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | (OK) |
| 113 | 1.1.255 | 1.1.1 | T_ACK (S=11) |  |  |
| 114 | 1.1.255 | 1.1.1 | MemoryWrite (S=12) | Count=63, Address=$60BD, Data=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | Write (Chunk 4) |
| 115 | 1.1.1 | 1.1.255 | T_ACK (S=12) |  |  |
| 116 | 1.1.1 | 1.1.255 | MemoryResponse (S=12) | Count=63, Address=$60BD, Data=FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF FF | (OK) |
| 117 | 1.1.255 | 1.1.1 | T_ACK (S=12) |  |  |
| 118 | 1.1.255 | 1.1.1 | MemoryWrite (S=13) | Count=4, Address=$60FC, Data=FF FF FF FF | Write (Chunk 5) |
| 119 | 1.1.1 | 1.1.255 | T_ACK (S=13) |  |  |
| 120 | 1.1.1 | 1.1.255 | MemoryResponse (S=13) | Count=4, Address=$60FC, Data=FF FF FF FF | (OK) |
| 121 | 1.1.255 | 1.1.1 | T_ACK (S=13) |  |  |
| 122 | 1.1.255 | 1.1.1 | PropertyValueRead (S=14) | ObjectIndex=3, PropertyId=7, Count=1, StartIndex=1 | Get Object table start address |
| 123 | 1.1.1 | 1.1.255 | T_ACK (S=14) |  |  |
| 124 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=14) | ObjectIndex=3, PropertyId=7, Count=1, StartIndex=1, Data=00 00 80 00 | (Start address = $8000) |
| 125 | 1.1.255 | 1.1.1 | T_ACK (S=14) |  |  |
| 126 | 1.1.255 | 1.1.1 | PropertyValueRead (S=15) | ObjectIndex=0, PropertyId=14, Count=1, StartIndex=1 | Get device status |
| 127 | 1.1.1 | 1.1.255 | T_ACK (S=15) |  |  |
| 128 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=15) | ObjectIndex=0, PropertyId=14, Count=1, StartIndex=1, Data=$04 | (OK) |
| 129 | 1.1.255 | 1.1.1 | T_ACK (S=15) |  |  |
| 130 | 1.1.255 | 1.1.1 | MemoryWrite (S=0) | Count=63, Address=$8000, Data=00 47 47 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 47 00 47 03 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 47 00 47 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 | Write (Sequence number starts again with 0) |
| 131 | 1.1.1 | 1.1.255 | T_ACK (S=0) |  |  |
| 132 | 1.1.1 | 1.1.255 | MemoryResponse (S=0) | Count=63, Address=$8000, Data=00 47 47 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 47 00 47 03 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 47 00 47 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 | (OK) |
| 133 | 1.1.255 | 1.1.1 | T_ACK (S=0) |  |  |
| 134 | 1.1.255 | 1.1.1 | MemoryWrite (S=1) | Count=63, Address=$803F, Data=00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 00 00 | Write |
| 135 | 1.1.1 | 1.1.255 | T_ACK (S=1) |  |  |
| 136 | 1.1.1 | 1.1.255 | MemoryResponse (S=1) | Count=63, Address=$803F, Data=00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 00 00 | (OK) |
| 137 | 1.1.255 | 1.1.1 | T_ACK (S=1) |  |  |
| 138 | 1.1.255 | 1.1.1 | MemoryWrite (S=2) | Count=18, Address=$807E, Data=00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 | Write |
| 139 | 1.1.1 | 1.1.255 | T_ACK (S=2) |  |  |
| 140 | 1.1.1 | 1.1.255 | MemoryResponse (S=2) | Count=18, Address=$807E, Data=00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 00 43 00 | (OK) |
| 141 | 1.1.255 | 1.1.1 | T_ACK (S=2) |  |  |
| 142 | 1.1.255 | 1.1.1 | PropertyValueRead (S=3) | ObjectIndex=2, PropertyId=7, Count=1, StartIndex=1 | Get Association table start address |
| 143 | 1.1.1 | 1.1.255 | T_ACK (S=3) |  |  |
| 144 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=3) | ObjectIndex=2, PropertyId=7, Count=1, StartIndex=1, Data=00 00 C0 00 | (Start address = $C000) |
| 145 | 1.1.255 | 1.1.1 | T_ACK (S=3) |  |  |
| 146 | 1.1.255 | 1.1.1 | MemoryWrite (S=4) | Count=22, Address=$C000, Data=00 05 00 01 00 01 00 02 00 0B 00 03 00 0C 00 04 00 15 00 05 00 16 | Write |
| 147 | 1.1.1 | 1.1.255 | T_ACK (S=4) |  |  |
| 148 | 1.1.1 | 1.1.255 | MemoryResponse (S=4) | Count=22, Address=$C000, Data=00 05 00 01 00 01 00 02 00 0B 00 03 00 0C 00 04 00 15 00 05 00 16 | (OK) |
| 149 | 1.1.255 | 1.1.1 | T_ACK (S=4) |  |  |
| 150 | 1.1.255 | 1.1.1 | PropertyValueRead (S=5) | ObjectIndex=1, PropertyId=7, Count=1, StartIndex=1 | Get Address table start address |
| 151 | 1.1.1 | 1.1.255 | T_ACK (S=5) |  |  |
| 152 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=5) | ObjectIndex=1, PropertyId=7, Count=1, StartIndex=1, Data=00 00 A0 00 | (Start address = $A000) |
| 153 | 1.1.255 | 1.1.1 | T_ACK (S=5) |  |  |
| 154 | 1.1.255 | 1.1.1 | MemoryWrite (S=6) | Count=12, Address=$A000, Data=00 05 19 01 21 01 22 01 29 01 2A 01 | Write |
| 155 | 1.1.1 | 1.1.255 | T_ACK (S=6) |  |  |
| 156 | 1.1.1 | 1.1.255 | MemoryResponse (S=6) | Count=12, Address=$A000, Data=00 05 19 01 21 01 22 01 29 01 2A 01 | (OK) |
| 157 | 1.1.255 | 1.1.1 | T_ACK (S=6) |  |  |
| 158 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=7) | ObjectIndex=4, PropertyId=13, Count=1, StartIndex=1, Data=00 FA 02 04 24 | Set application program version |
| 159 | 1.1.1 | 1.1.255 | T_ACK (S=7) |  |  |
| 160 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=7) | ObjectIndex=4, PropertyId=13, Count=1, StartIndex=1, Data=00 FA 02 04 24 | (OK) |
| 161 | 1.1.255 | 1.1.1 | T_ACK (S=7) |  |  |
| 162 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=8) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=02 00 00 00 00 00 00 00 00 00 | Terminate loading application program |
| 163 | 1.1.1 | 1.1.255 | T_ACK (S=8) |  |  |
| 164 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=8) | ObjectIndex=4, PropertyId=5, Count=1, StartIndex=1, Data=$01 | (OK) |
| 165 | 1.1.255 | 1.1.1 | T_ACK (S=8) |  |  |
| 166 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=9) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=02 00 00 00 00 00 00 00 00 00 | Terminate loading Object table |
| 167 | 1.1.1 | 1.1.255 | T_ACK (S=9) |  |  |
| 168 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=9) | ObjectIndex=3, PropertyId=5, Count=1, StartIndex=1, Data=$01 | (OK) |
| 169 | 1.1.255 | 1.1.1 | T_ACK (S=9) |  |  |
| 170 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=10) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=02 00 00 00 00 00 00 00 00 00 | Terminate loading Association table |
| 171 | 1.1.1 | 1.1.255 | T_ACK (S=10) |  |  |
| 172 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=10) | ObjectIndex=2, PropertyId=5, Count=1, StartIndex=1, Data=$01 | (OK) |
| 173 | 1.1.255 | 1.1.1 | T_ACK (S=10) |  |  |
| 174 | 1.1.255 | 1.1.1 | PropertyValueWrite (S=11) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=02 00 00 00 00 00 00 00 00 00 | Terminate loading Address table |
| 175 | 1.1.1 | 1.1.255 | T_ACK (S=11) |  |  |
| 176 | 1.1.1 | 1.1.255 | PropertyValueResponse (S=11) | ObjectIndex=1, PropertyId=5, Count=1, StartIndex=1, Data=$01 | (OK) |
| 177 | 1.1.255 | 1.1.1 | T_ACK (S=11) |  |  |
| 178 | 1.1.255 | 1.1.1 | Restart (S=12) |  | Restart |
| 179 | 1.1.255 | 1.1.1 | T_Disconnect |  | Terminate TL session |

### Acknowledgment of telegrams by Link Layer

Figure 7: Positive acknowledgement

To ensure transmission acknowledgement, it is necessary that the Link Layer of the addressed KNX bus devices or the line -/ backbone coupler immediately sends an acknowledgement all within a specified time frame. This is because each device expects after sending a telegram, to receive a LL acknowledgement. The following possibilities exist:

| Type | Abbr. | Coding | Explanation |
| --- | --- | --- | --- |
| Positive Acknowledgement | LL_ACK | CC (hex) 1100 1100 | The sender of the telegram will not repeat the telegram |
| Negative Acknowledgement | LL_NAK | 0C (hex) 0000 1100 | The sender of the telegram will repeat the telegram |
|  | LL_BUSY | C0 (hex) 1100 0000 |  |
| No reaction |  |  | The sender of the telegram will repeat the telegram |

The LL acknowledgements are 1 octet long.

#### Positive Acknowledgement (LL_ACK)

Confirmation that:

The telegram was received without any errors and

That the addressed device indeed had the individual address contained as destination address in the received telegram (in case of connection-oriented communication) or

That the addressed device indeed listens to the group address contained as destination address in the received telegram (in case of connectionless communication).

#### Negative Acknowledgement (LL_NAK)

- Confirmation that:

Confirmation that the received telegram was received with errors

The sender of the telegram will repeat the telegram (maximum number of repetitions is fixed in the software of KNX devices)

#### Busy Acknowledgement (LL_BUSY)

- Confirmation that:

the received telegram could not be processed by LL as the buffers are full

The sender of the telegram will repeat the telegram (number of repetitions is fixed by the developer in the application program and cannot be changed by the installer)

#### Multiple LL Acknowledgements

Because of multicast communication, several devices can at the same time send a LL acknowledgement as a reaction to the same telegram. This is called a group acknowledgement. Because of collision avoidance (See §0), only one LL acknowledgement will be observed by the sender.

#### Priority LL Acknowledgements

It is possible, that several receivers send a different acknowledgement, e.g., in the case where receivers simultaneously send an ACK, NAK or BUSY.

First of all, it is important to know the LL acknowledgement priority, which is:
[highest priority] LL_BUSY > LL_NAK > LL_ACK [lowest priority].

Example: suppose that 10 devices are addressed via the same multicast telegram, where 9 of them react with LL_ACK and one with LL_NAK. Because of collision avoidance only the LL_NAK will be observed by the sender, which will make the sender repeat the telegram.

Good to know:

The sender cannot detect which device did not send a LL_ACK

Receivers that can already process the first (non-repeated) multicast telegram correctly will send an ACK (sender will however observe a NAK) and will pass the telegram data on to the upper layers (e.g., a relay will be switched on)

As the sender has received a NAK, it will now repeat the telegram with the repetition flag set (Control field 1, D5 = 0). The receivers that already reacted with an ACK, will send an ACK again, but the telegram data will not be passed on to the upper layers.

#### LL Acknowledgements in case there are couplers

##### Sender and receiver are in the same line

If sender and destination device are in the same line, then LL acknowledgements are sent by the destination device directly, possible repetitions are sent by the sender directly.

##### Sender and receiver are on opposite side of a coupler

The sender will communicate with the coupler and will also receive the LL acknowledgments from the coupler. The same coupler will communicate with the destination device (from the opposite side) and will also receive the LL acknowledgements from the destination device. And if required, repetitions are sent by the coupler.

##### Sender and receiver are on opposite side of two couplers

The sender will communicate with the first coupler and will also receive the LL acknowledgments from the first coupler. The first coupler will communicate with the second coupler. The second coupler will communicate with the destination device and will also receive the LL acknowledgements from the destination device. And if required, repetitions are sent by the second coupler.

## Physical Layer

The lowest layer in the OSI Reference Model is called the Physical Layer. This layer has the task of identifying the bits received from the Link Layer, converting them to physical signals such as voltages, currents and electromagnetic waves (radio or optical signals) and finally transmitting these signals over the bus transmission media (copper cable, optical fiber etc.).

Principal aim of the services provided by the Physical Layer is to shield the Link Layer from the physical means used for bit transmission. It is thus ensured that the upper layers of the network remain independent of the transmission physics used, so that principally the transmission media could be changed without having any effect upon the upper layers.

The Physical Layer contains the protocol, defines the cabling and wiring between devices and contains the specifications for electromechanical components such as plugs and connectors.

### Serial characters

This paragraph is about the serial characters based on the octets as described above. In this context, the following aspects need to be considered:

In order to send such an octet over the TP medium, 3 additional bits need to be sent along. This means that for each octet of a TP1 telegram in total 8 + 3 = 11 bits are sent. Each such 11-bit constellation is called a serial character.

The bits of the telegrams are sent character by character.

The data bits of each octet are sent in reverse order, i.e., from least significant bit (LSB) to most significant bit (MSB)

The Start bit has the logical value 0 and is sent before the LSB of the octet.

The Parity bit has the logical value which equals to the even parity over the 8 bits of the octet. It is sent after the MSB of the octet.

The Stop bit has the logical value ‘1’ and is sent after the Parity bit

Figure 8: Difference between Serial character and octet

### Transmission time on TP1

As mentioned in clause  3.7.1, the bits of the telegram are sent character by character. Each serial character is 11 bits. A pause t4 which is 2 bits long, separates one character from the next. This means, that a total of 13 bit periods is required per serial character.

The protocol foresees a transmission break t2 of at least one serial character width after the sending of the entire telegram. The next serial character width is reserved for the reception of the acknowledgement (ACK, NAK or BUSY), which must be sent by the Link Layer of the addressed bus devices and/or by the coupler (see 3.6.4.6 for more information).

After the acknowledgement has been transmitted, a bus idle time t3 of 50-bit periods must be observed for telegrams with system priority. A further 3-bit period (t1) are added for the lower priorities.

The total time ttot required for the transmission of the telegram and the acknowledgement and the observance of the bus idle time is calculated as follows:

### Collision Avoidance: Bus access control on TP1

Telegrams coming from devices connected to the same TP1 segment and sending at the same time would collide without the collision avoidance technology that all these devices support.

In KNX TP1, the bus access is not controlled centrally according to the master/slave principle but according to the CSMA/CA procedure (Carrier Sense Multiple Access with Collision Avoidance).

The picture below explains how this works: device A and device B send telegrams at the same time. In fact, this means that:

|  | the TP1-receive part (Rx) of device B will monitor the TP1-send part (Tx) of device A the TP1-receive part (Rx) of device A will monitor the TP1-send part (Tx) of device B |
| --- | --- |

The following rules apply:

both A and B continue sending if:

A and B are sending and hearing identical signals

A continues sending and B stops sending when:

  - B hears an active signal while not sending itself an active signal

B continues sending and A stops sending when:

  - A hears an active signal while not sending itself an active signal

#### Example

Device A sends a LL_ACK and device B sends a LL_NAK (at the same time).

LL_ACK = CC (hex) = 1100 1100 (binary)

LL_NAK = 0C (hex) = 0000 1100 (binary)

The signal for the start bit does not need to be considered because they are the same for all telegrams, then also note that when sending, the bits of the octets are sent in reversed order.

As a “bit” can only have two logical states, i.e., ‘0’ and ‘1’, this will mean that if several bus devices transmit simultaneously, the logical state ‘0’ will prevail over the logical state ‘1’.

- Logical state ‘1’: passive signal

- Logical state ‘0’: active signal

This gives the following situation:

|  | D0 | D1 | D2 | D3 | D4 | D5 | D6 | D7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Device A | 0 | 0 | 1 | 1 | 0 | 0 | 1 | 1 |
| Device B | 0 | 0 | 1 | 1 | 0 | 0 | 0 | 0 |
| Signal | = | = | = | = | = | = | B > A |  |

From D0 till D5 the signals are the same; both devices keep on sending

At D6, A receives from B an active signal and B receives from A, a passive signal

B keeps on sending, which means that the LL_NAK will appear on the bus

A stops sending, which means that the LL_ACK will NOT appear on the bus

Consequently, the negative link layer acknowledgement appears on the bus, meaning that one of the devices  has detected an error in the received frame. With this LL_NAK, the original sender will repeat its original sent frame, now with the repetition flag set in the control byte. Because of this, this telegram will have precedence of a non-repeated telegram, should one be sent at the same time.

### Signal shape on TP1

The bit pulses of TP1 telegrams have the following shapes:

Figure 9: Bit pulse of TP1 telegram

Details:

For ‘1’ & ‘0’

- DC-level: 21...32 V

- t: 104µs

Only for ‘0’

- Voltage drop: 3...10 V

- Voltage overshoot: max 13 V

- tdrop: 35µs

#### Active vs. passive signals

A signal with shape '1' is called a passive signal.

A signal with shape '0' is because of the voltage drop (and overshoot) called an active signal.

Bit representation in detail:

Figure 10: "0"-Bit character

| Parameter / Point | min | max |
| --- | --- | --- |
| bit-time | 1/9600 s (type) |  |
| tactive | 35 µs (type) |  |
| ts (time between Ua > A and Ua > B) | 25 µs | 70 µs |
| time (Point D - E) | 50 µs |  |
| voltage (DC-Part) | 21 V | 32 V |
| voltage Ua (Point A) concerning Ref | - 0,7 V | - 10,5 V |
| voltage Ua (Point B) concerning Ref | - 0,5 V | - 10,5 V |
| voltage Ue (Point C - D) concerning Ref | 0 V | +13 V |
| voltage Uend (Point F) concerning Ref | - 0,35 V | +1,8 V |

Figure 11: "1"-Bit Character

| Parameter | Value |
| --- | --- |
| bit-time | 1/9600 s |
| voltage (DC-Part) | 21 VDC to 32 VDC |
| voltage Up | max 0,3 V |
| voltage Un | max - 2 V |
| slopes (AC-Part) | max. 400 mV/ms |

Transmission speed:

9.6 Kbit/s

Transmission delay:

To ensure collision detection and correction, the signal propagation delay between sender an any receiver must not exceed 10µs This condition is automatically met if you

use only authorized bus cables

observe the maximum distance of 700 meters between any two bus devices and

do not install more than 256 bus devices in one electrical segment

Bus coupling method:

Repeater, symmetrical

Connection techniques used for cables:

Bus connector with 4-wire connections per conductor, for connection to the bus device.

Technical data of an electrical segment:

Random topology.

Total capacitance of one segment:
without bus devices, line coupler, line repeater: 100 nF max.
with bus devices, line coupler, line repeater: 120 nF max.
(measured at 10 KHz)

Bus line resistance between power supply and bus device, line coupler or line repeater: 25  max.
Bus line resistance between two bus devices, line couplers or line repeaters: 50  max.

Minimum resistance between two power supplies: 15 
Minimum bus line length between two power supplies: 200 m (or see manufacturer instruction).

Voltage drop on bus line between power supply and bus device or line coupler: 5 V max.
This criterion determines how the bus devices may be physically arranged in the bus line of a segment. It is not permitted, for instance, to have a conglomeration of bus devices installed at one end of a 350 m bus line and the power supply at the other.

For max. length line segment, max. length between two bus devices and max. length between power supply and bus device refer, see Basic course documentation.

Max. number of bus devices in an electrical segment see Basic course documentation.

Technical data of the bus line:

| Type | Twisted Pair, two pairs |
| --- | --- |
| Line resistance per line segment | maximum 37 /km (Loop resistance max. 75 /km) |
| Load capacitance line / line | max 100 nF/km (800 Hz) |
| Screening | Thin strip with drain terminal |
| Number of twists | minimum 5 twists /m |
| Line diameter/cross section | 0.8 mm |

TP cable that meets the KNX requirements in Volume 9 of the KNX specifications (e.g., YCYM 2x2x0.8 or J-Y(St)Y 2x2x0.8) can be recognized (without KNX logo) or certified (with KNX logo) by KNX Association. Only the green standard TP cable guarantees the cable lengths as mentioned above. The maximum length of all other cables per line is indicated in the respective cable data sheet.

Transmission method used:

Time division multiplexing, baseband, symmetrical

# Appendix: Further examples of cEMI telegrams

- 29 00 BC E0 11 1B 01 17 01 00 81

| 1 | 29 |  | From bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11 1B | 0001 0001 0001 1011 | D0-D7 0001 1011 = Device address (27) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.27 |
| 8-9 | 01 17 | 0000 0001 0001 0111 | D7 of Control field 2 = 1 group address At least one bit = 1 group address (multicast) D0-D7 = 23 D0-D2 = 1 D3-D7 = 0 Group address = 0/1/23 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 00 | 0000 0000 | 00: Unnumbered data package 0000 = don’t care 00 = APCI |
|  | 81 | 1000 0001 | 10 = APCI The APCI is thus = 0010, meaning a GroupValueWrite 00 0001 = data To Group address 0/1/23 with value = 1 Explanation: the device 1.1.27 sends a GroupValueWrite via the group address 0/1/23 with the value 1. |

- 29 00 BC E0 11 1E 18 04 03 00 80 18 0D

| 1 | 29 |  | From bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11 1E | 0001 0001 0001 1110 | D0-D7 0001 1110 = Device address (30) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.30 |
| 8-9 | 18 04 | 0001 1000 0000 0100 | D7 of Control field 2 = 1 group address At least one bit = 1 group address (multicast) D0-D7 = 4 D0-D2 = 0 D3-D7 = 3 Group address = 3/0/4 |
| 10 | 03 | 0000 0011 | L > 0 data package |
|  | 00 | 0000 0000 | 00: Unnumbered data package 0000 = don’t care 00 = APCI |
|  | 80 | 1000 0000 | 10 = APCI 00 0000 = data |
|  | 18 | 0001 1000 | 0001 1000 = data |
|  | 0D | 0000 1101 | 0000 1101 = data The APCI is thus = 0010, meaning a GroupValueWrite to Group Address 1/1/4 with the following data: 0001 1000 0000 1101. If this data is encoded according to (DPT_Value_Wsp - 9.005) (Windspeed), then this means the following: (0.01 x 13) x 23 = 1,04 m/s |

- 2E 00 BC E0 11 FF 02 04 01 00 8B

| 1 | 2E |  | Sent to bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11 FF | 0001 0001 1111 1111 | D0-D7 1111 1111 = Device address (255) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.255 (interface) |
| 8-9 | 02 04 | 0000 0010 0000 0100 | D7 of Control field 2 = 1 group address At least one bit = 1 group address (multicast) D0-D7 = 4 D0-D2 = 2 D3-D7 = 0 Group address = 0/2/4 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 00 | 0000 0000 | 00: Unnumbered data package 0000 = don’t care 00 = APCI |
|  | 8B | 1000 1011 | 10 = APCI 00 1011 = data The APCI is thus = 0010, meaning a GroupValueWrite to Group Address 0/2/4 with the data 00 1011: If this data is encoded according to (DPT_Control_Dimming - 3.007), then this means the following: 00 1011 (DPT_3.007) Dimming increase, step code 3, meaning 4 = 2 to the power (stepcode 3 minus 1) or 100% divided by 4 = 25% or Dim up: 25% |

- 29 00 BC E0 11 11 05 04 02 00 40 C0

| 1 | 29 |  | From bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | BC | 1011 1100 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 11 Low priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | E0 | 1110 0000 | D7 = 1 Address type = group address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11 11 | 0001 0001 0001 0001 | D0-D7 0001 0001 = Device address (17) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.17 |
| 8-9 | 05 04 | 0000 0101 0000 0100 | D7 of Control field 2 = 1 group address At least one bit = 1 group address (multicast) D0-D7 = 4 D0-D2 = 5 D3-D7 = 0 Group address = 0/5/4 |
| 10 | 02 | 0000 0010 | L > 0 data package |
|  | 00 | 0000 0000 | 00: Unnumbered data package 0000 = don’t care 00 = APCI |
|  | 40 | 0100 0000 | 01 = APCI 00 0000 = data |
|  | C0 | 1100 0000 | The APCI is thus = 0001, meaning a GroupValueResponse from Group Address 0/5/4 with the following data: 1100 0000 If this data is encoded according to (DPT_Scaling - 5.001), then this means the following: 192255* 100 = 75,2 % dimming value |

- 2E 00 B0 60 11 FF 11 1B 01 43 00

| 1 | 2E |  | Sent to the bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11 FF | 0001 0001 1111 1111 | D0-D7 1111 1111 = Device address (255) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.255 (interface) |
| 8-9 | 11 1B | 0001 0001 0001 1011 | D7 of Control field 2 = 0 individual address At least one bit = 1 individual address (unicast) D0-D7 = 27 D0-D3 = 1 D4-D7 = 1 Individual address = 1.1.27 |
| 10 | 01 | 0000 0001 | L > 0 data package |
|  | 43 | 0100 0011 | 01: Numbered data package 00 00 = Sequence number (0) 11 = APCI |
|  | 00 | 0000 0000 | 00 = APCI the APCI is thus 1100, meaning a DeviceDescriptorRead with data 00 0000 Explanation: the device with address 1.1.255 requests from the device with address 1.1.27 to send back its device descriptor, which will indicate the medium and the system profile supported by that device. |

- 29 00 B0 60 11 1B 11 FF 03 43 40 07 B0

| 1 | 29 |  | From the bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Additional Information - does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11 1B | 0001 0001 0001 1011 | D0-D7 0001 1011 = Device address (27) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.27 |
| 8-9 | 11 FF | 0001 0001 1111 1111 | D7 of Control field 2 = 0 individual address At least one bit = 1 individual address (unicast) D0-D7 = 255 D0-D3 = 1 D0-D3 = 1 Individual address = 1.1.255 |
| 10 | 03 | 0000 0011 | L > 0 data package |
|  | 43 | 0100 0011 | 01: Numbered data package 00 00 = Sequence number (0) 11 = APCI |
|  | 40 | 0100 0000 | 01 = APCI The APCI is thus 1101, meaning a DeviceDescriptorResponse with data 07B0, which indicates TP1 as medium, System profile System B. |

- 2E 00 B0 60 11 FF 11 1B 00 81

| 1 | 2E |  | Sent to the bus |
| --- | --- | --- | --- |
| 2 | 00 |  | TP or IP telegram |
| 3 | - |  | Does not exist |
| 4 | B0 | 1011 0000 | D7 = 1 Standard frame D6 = 0 D5 = 1 not repeated frame D4 = 1 Regular broadcast applied, TP or IP telegram D3-D2 = 00 System priority D1 = 0 LL_ACK is not requested for the telegram D0 = 0 telegram contains no errors |
| 5 | 60 | 0110 0000 | D7 = 0 Address type = individual address D6-D4 Hop count = 6 D3-D0 0000 (don’t care) |
| 6-7 | 11 FF | 0001 0001 1111 1111 | D0-D7 1111 1111 = Device address (255) D0-D3 0001 = Line address (1) D0-D3 0001 = Area address (1) Source address = 1.1.255 (interface) |
| 8-9 | 11 1B | 0001 0001 0001 1011 | D7 of Control field 2 = 0 individual address At least one bit = 1 individual address (unicast) D0-D7 = 27 D0-D3 = 1 D0-D3 = 1 Individual address = 1.1.27 |
| 10 | 00 | 0000 0000 | L = 0 control data |
|  | 81 | 1000 0001 | 10: Unnumbered control data 00 00 = don’t care 01 = T_Disconnect Explanation: the device with individual address 1.1.255 terminates its Transport Layer connection with the device with individual address 1.1.27. |

These are flags in the RAM of a device. These flags shall not be confused with the group object flags or Download flags

Also referred to as routing counter

Parity checks is a technique of detecting errors in binary transmissions. It is based on sending redundant data to verify the integrity of the received data. To use a parity check, at the transmission end, the data is divided into groups of bits. For each group, a parity bit is generated and sent along with the data group. At the receiving end, another parity bit is generated based on the received data and compared with the parity bit sent by the sender. If the parity bits match, the data are considered (probably) valid, but if the parity bits do not match, an error has occurred during transmission.

Most modern interfaces now support cEMI, however in the past interfaces supported other External Message Interfaces, i.e., EMI1 and EMI2

Also referred to as PEI-program: Certain bus devices have two application programs. This type of architecture is required when an application program should control the PEI, i.e., the interface between a BCU and the Application module. Application program 2 is thus a separate program from the actual application program which lays down the actual functionality of a KNX device (Application program).

The DPT information is not sent on the bus.